PWM Baseline Verification

PWM baseline output was verified on PIC18F4620 drum machine hardware using PC based oscilloscope. The PWM channels were configured with:

Fosc = 40 MHz

PR2 = 0x7F

Timer2 Prescale = 1

Expected Carrier Frequency:

FPWM = FOSC / 4(PR2 + 1) x prescale => 40 x 106 / 4(127 + 1) x 1 = 78.125 KHz

Expected Period:

TPWM = 1 / FPWM => 1/78125 => 12.8 us

With oscilloscope set to 50.00 us/div the expected cycles per division are:

50.00 us / 12.8 us => 3.906 cycles per division

In the figure below we see just under 4 cycles per division